# Technical Report: Dataset Refinement & Optimization Strategy

From: Keerthana (Lead Programmer)
Subject: Refinement of Sensor Fusion Dataset for AI Model Training and XAI Logic

### 1. Executive Summary

After analyzing the initial 100-row dataset, I have identified critical areas for refinement to ensure the Isolation Forest model achieves high accuracy and the XAI (Explainable AI) logic remains physically consistent. The current dataset size is insufficient for robust training, and certain sensor correlations need to be mathematically aligned.

### 2. Identified Gaps & Refinements

#### A. Data Volume Expansion

- Issue: 100 rows is statistically insufficient for an Isolation Forest to distinguish between "Global Outliers" and "Local Anomalies."

- Refinement: We must expand the dataset to a minimum of 1,000–1,500 rows.

- Strategy: Utilize a script-based Synthetic Data Generator to maintain a 70/30 distribution (70% Normal, 30% Faults) to prevent the model from becoming biased toward any single fault.

#### B. Physics-Based Correlation (The "Power-Current" Rule)

- Issue: In the current data, Power and Current are treated as independent random variables.

- Refinement: In industrial fans,

Power≈Voltage×Current×Power FactorPower≈Voltage×Current×Power Factor

.

- Action: We need to normalize the Power column to be a function of Current. This ensures that if the model sees High Current but Low Power, it correctly identifies it as a Sensor Malfunction rather than a Fan Fault.

#### C. Multi-Sensor Synergy for XAI

To make Harshine’s Explainable AI part credible, the data must follow "Sensor Fusion" rules:

- Bearing Fault: Must show a simultaneous spike in vibration AND temperature.

- Obstruction: Must show high sound while vibration remains relatively stable.

- Blade Unbalance: Must show high vibration but normal temperature.

- Demand-Controlled Ventilation: co_level and nox_level must correlate with power usage (as speed increases to clear gases, power consumption should follow the Cube Law).

### 3. Proposed Feature Engineering (For the Model)

I suggest adding a "Health Score" or "Energy Efficiency Index" feature during the training phase.

- Energy Optimization Logic: I have already developed the logic based on the Fan Affinity Laws. We need the dataset to reflect that when co_level is low, the current draw is also minimized.